MICROPROCESSOR AND EMBEDDED SYSTEMS

MULTIPLE CHOICE QUESTIONS

UNIT-1

|  | 8086 can access up to? |  |  |  |
| --- | --- | --- | --- | --- |
|  | A) | 512KB | B) | 2Mb |
|  | C) | 1Mb | D) | 256KB |
|  | 8086 has ___ address bus |  |  |  |
|  | A) | 20 bit | B) | 8 bit |
|  | C) | 16 bit | D) | 24 bit |
|  | Which flag is set to 1 when the result of arithmetic or logical operation is zero else it is set to 0? |  |  |  |
|  | A) | Trap flag | B) | Zero flag |
|  | C) | Sign flag | D) | Over flow flag |
|  | Which index register in the 8086 is primarily used for operations involving string processing? |  |  |  |
|  | A) | BP, SP | B) | DH, DL |
|  | C) | BH, BL | D) | SI, DI |
|  | The different ways in which a source operand is denoted in an instruction is known as |  |  |  |
|  | A) | Instruction Set | B) | Interrupts |
|  | C) | 8086 Configuration | D) | Addressing Modes |
|  | The 16 bit flag of 8086 microprocessor is responsible to indicate ___________ |  |  |  |
|  | A) | the condition of result of ALU operation | B) | the condition of memory |
|  | C) | the result of addition | D) | the result of subtraction |
|  | The 1 MB byte of memory can be divided into ______ segment |  |  |  |
|  | A) | 1 Kbyte | B) | 64 Kbyte |
|  | C) | 34 Kbyte | D) | 256 Kbyte |
|  | The SS is called as ________ |  |  |  |
|  | A) | stack segment | B) | random stack |
|  | C) | single stack | D) | sequence stack |
|  | The BIU contains FIFO/ INSTRUCTION QUEUE register of size __________ bytes |  |  |  |
|  | A) | 8 | B) | 6 |
|  | C) | 4 | D) | 12 |
|  | The instruction, MOV AX, 0005H belongs to the address mode |  |  |  |
|  | A) | register | B) | direct |
|  | C) | immediate | D) | register indirect |
|  | The instruction, MOV AX, [2500H] is an example of |  |  |  |
|  | A) | immediate addressing mode | B) | direct addressing mode |
|  | C) | indirect addressing mode | D) | register addressing mode |
|  | The instruction, MOV AX,[BX] is an example of |  |  |  |
|  | A) | direct addressing mode | B) | register addressing mode |
|  | C) | register relative addressing mode | D) | register indirect addressing mode |
|  | If the location to which the control is to be transferred lies in a different segment other than the current one, then the mode is called |  |  |  |
|  | A) | intrasegment mode | B) | intersegment direct mode |
|  | C) | intersegment direct and indirect mode | D) | intersegment indirect mode |
|  | If [CS]=348AH, [IP]=4214H, then the 20-bit physical address from which the code is accessed will be |  |  |  |
|  | A) | 455CAH | B) | 0769EH |
|  | C) | 390B4H | D) | 38AB4H |
|  | Setting of direction flag in 8086 causes, |  |  |  |
|  | A) | The microprocessor to operate in single stepping mode | B) | The microprocessor to operate in break point mode |
|  | C) | The microprocessor string operations in auto increment mode | D) | The microprocessor string operations in auto decrement mode |
|  | In the 8086 microprocessor, if a program execution requires accessing an array stored in memory with an offset stored in SI (Source Index), which combination of registers would typically be used for addressing? |  |  |  |
|  | A) | CS:SI (Code Segment with Source Index) | B) | DS:SI (Data Segment with Source Index) |
|  | C) | SS:SI (Stack Segment with Source Index) | D) | ES:SI (Extra Segment with Source Index) |
|  | The size of ALU in 8086 is |  |  |  |
|  | A) | 8 bit | B) | 16 bit |
|  | C) | 32 bit | D) | 64 bit |
|  | The directive used to inform the assembler, the names of the logical segments to be assumed for different segments used in the program is |  |  |  |
|  | A) | ASSUME | B) | SEGMENT |
|  | C) | SHORT | D) | DB |
|  | Match the following a) DB 1) used to direct the assembler to reserve only 10-bytes b) DT 2) used to direct the assembler to reserve only 4 words c) DW 3) used to direct the assembler to reserve byte or bytes d) DQ 4) used to direct the assembler to reserve words |  |  |  |
|  | A) | a-3, b-2, c-4, d-1 | B) | a-2, b-3, c-1, d-4 |
|  | C) | a-3, b-1, c-2, d-4 | D) | a-3, b-1, c-4, d-2 |
|  | The directive that marks the end of a logical segment is |  |  |  |
|  | A) | ENDS | B) | END |
|  | C) | ENDS & END | D) | ENDD |

|  | The instruction that is invalid among the following is |  |  |  |
| --- | --- | --- | --- | --- |
|  | A) | MOV AX, BX | B) | MOV AX, [BX] |
|  | C) | MOV 55H, BL | D) | MOV AL, 55H |
|  | What does the RCR instruction do? |  |  |  |
|  | A) | Rotates bits to the right including the Carry Flag | B) | Rotates bits to the left excluding the Carry Flag |
|  | C) | Shifts bits to the right | D) | Performs logical AND operation |
|  | The JS is called as |  |  |  |
|  | A) | jump signed bit | B) | jump single bit |
|  | C) | jump simple bit | D) | jump signal it |
|  | The expansion of DAA is |  |  |  |
|  | A) | decimal adjust after addition | B) | decimal adjust before addition |
|  | C) | decimal adjust accumulator | D) | decimal adjust auxiliary |
|  | Which group of instructions do not affect the flags? |  |  |  |
|  | A) | Arithmetic operations | B) | Logic operations |
|  | C) | Data transfer operations | D) | Branch operations |
|  | The LOOP instruction in 8086: |  |  |  |
|  | A) | Jumps if CX is zero | B) | Decrements CX and jumps if CX ≠ 0 |
|  | C) | Increments CX and jumps | D) | Decrements CX and jumps if CX = 0 |
|  | The instruction RET is used to: |  |  |  |
|  | A) | Return from subroutine | B) | Jump to a memory address |
|  | C) | Load a segment register | D) | Move to the next instruction |
|  | The JMP instruction is used to: |  |  |  |
|  | A) | Jump to a specified location | B) | Call a subroutine |
|  | C) | Return from subroutine | D) | Interrupt execution |
|  | What is the effect of CMP AX, BX? |  |  |  |
|  | A) | Adds AX and BX | B) | Subtracts BX from AX |
|  | C) | Moves BX to AX | D) | Subtracts AX from BX |
|  | What does PUSH AX do in 8086? |  |  |  |
|  | A) | Moves AX into memory | B) | Moves AX onto the stack |
|  | C) | Moves AX into the instruction pointer | D) | Moves AX to an I/O port |
|  | If AX = 36H and AAM is executed, what will be the values of AH and AL? |  |  |  |
|  | A) | AH = 03H, AL = 06H | B) | AH = 06H, AL = 03H |
|  | C) | AH = 02H, AL = 08H | D) | AH = 04H, AL = 06H |
|  | The instruction SAR BX, 1 does what? |  |  |  |
|  | A) | Shifts right with zero fill | B) | Shifts right and maintains the sign bit |
|  | C) | Shifts left | D) | Rotates right |
|  | The instruction PUSHF is used to: |  |  |  |
|  | A) | Push a register onto the stack | B) | Push the flag register onto the stack |
|  | C) | Pop a register from the stack | D) | Load a segment register |
|  | The IN AL, DX instruction: |  |  |  |
|  | A) | Inputs a byte from a port address in DX into AL | B) | Inputs a word from memory into AL |
|  | C) | Moves a byte from AL to DX | D) | Outputs a byte to a port |
|  | If AL = 05H, what will be the result of executing NEG AL? |  |  |  |
|  | A) | FAH | B) | FBH |
|  | C) | FCH | D) | FDH |
|  | If the DIV instruction is used on an 8-bit operand, which register stores the quotient? |  |  |  |
|  | A) | AX | B) | AL |
|  | C) | AH | D) | DX |
|  | If AL = 27H and ADD AL, 48H is executed, what will be the result after DAA? |  |  |  |
|  | A) | 75H and CF=1 | B) | 6FH and AF = 1 |
|  | C) | 75H and AF = 1 | D) | 6FH and CF = 1 |
|  | The instruction SHL AX, 1 performs: |  |  |  |
|  | A) | Divides AX by 2 | B) | Multiplies AX by 2 |
|  | C) | Rotates AX left | D) | Exchanges upper and lower bytes of AX |
|  | If AX = 1001 0110 1010 1100B and CF = 1, what will be the result of RCL AX, 1? |  |  |  |
|  | A) | 0010 1101 0101 1001B, CF = 1 | B) | 0010 1101 0101 1000B, CF = 1 |
|  | C) | 0010 1101 0101 1001B, CF = 0 | D) | 1001 0110 1010 1101B, CF = 1 |
|  | The AAM instruction operates on which register? |  |  |  |
|  | A) | AL | B) | AX |
|  | C) | AH | D) | DX |