MICROPROCESSOR AND EMBEDDED SYSTEMS

MULTIPLE CHOICE QUESTIONS

UNIT-3

|  | If MN/MX is low the 8086 operates in __________ mode |  |  |  |
| --- | --- | --- | --- | --- |
|  | A) | Minimum | B) | Maximum |
|  | C) | Minimum and Maximum | D) | Medium |
|  | Which pin is used in both minimum and maximum modes to latch the address from the multiplexed address/data lines? |  |  |  |
|  | A) | MN/MX | B) | ALE |
|  | C) | HLDA | D) | INTA |
|  | Which pin is used exclusively in maximum mode to indicate that the processor has granted a bus request? |  |  |  |
|  | A) | MN/MX | B) | INTR |
|  | C) | HLDA | D) | ALE |
|  | Which of the following pins is common to both minimum and maximum modes for external interrupt requests? |  |  |  |
|  | A) | HLDA | B) | MN/MX |
|  | C) | INTR | D) | S1 |
|  | Which pin is used by the 8086 to indicate that an external device is ready to exchange data? |  |  |  |
|  | A) | READY | B) | INTR |
|  | C) | ALE | D) | HLDA |
|  | The pin of minimum mode AD0-AD15 has ____________ data bus |  |  |  |
|  | A) | 16 bit | B) | 20 bit |
|  | C) | 32 bit | D) | 4 bit |
|  | During a read cycle, when are the address lines present on the multiplexed bus? |  |  |  |
|  | A) | After the data is transferred | B) | During the WR signal activation |
|  | C) | When the ALE signal is active | D) | When the HLDA signal is active |
|  | In register-to-register MOV instructions, which field within the machine code identifies the destination register? |  |  |  |
|  | A) | MOD field | B) | REG field |
|  | C) | R/M field | D) | Direction bit |
|  | In 8086 MOV instruction encoding, what does the 'MOD' field within the ModR/M byte represent? |  |  |  |
|  | A) | The specific register used | B) | The addressing mode for the memory operand |
|  | C) | The data size (byte/word) | D) | The immediate value provided |
|  | When encoding a MOV instruction that involves a memory operand, which additional byte is typically required? |  |  |  |
|  | A) | Immediate operand byte | B) | Function code byte |
|  | C) | Status byte | D) | Displacement byte |
|  | Which bit in the MOV instruction encoding specifies whether the data transfer is of a byte or a word? |  |  |  |
|  | A) | D (direction) bit | B) | MOD field bit |
|  | C) | W (word) bit | D) | REG field bit |
|  | Which instruction is used to generate a software interrupt in the 8086? |  |  |  |
|  | A) | INT | B) | INTA |
|  | C) | IRET | D) | PUSHF |
|  | Which pin does the 8086 use to acknowledge an external interrupt request? |  |  |  |
|  | A) | INTR | B) | NMI |
|  | C) | INTA | D) | HOLD |
|  | Which type of interrupt in the 8086 cannot be disabled by software? |  |  |  |
|  | A) | Software interrupt | B) | Maskable interrupt |
|  | C) | Non‐maskable interrupt | D) | Timer interrupt |
|  | Which instruction is used to return from an interrupt service routine in the 8086? |  |  |  |
|  | A) | RET | B) | IRET |
|  | C) | JMP | D) | CALL |
|  | During an interrupt response, which register is the first to be pushed onto the stack by the 8086? |  |  |  |
|  | A) | CS (Code Segment) | B) | IP (Instruction Pointer) |
|  | C) | FLAGS | D) | DS (Data Segment) |
|  | Example of an external interrupt is |  |  |  |
|  | A) | Divide by zero interrupt | B) | Keyboard interrupt |
|  | C) | Overflow interrupt | D) | Type2 interrupt |
|  | What is the primary function of the 8086 interrupt vector table? |  |  |  |
|  | A) | Store operands | B) | Hold interrupt service routine (ISR) addresses |
|  | C) | List I/O addresses | D) | Track cycles |
|  | How many entries are contained in the 8086 interrupt vector table? |  |  |  |
|  | A) | 64 entries | B) | 128 entries |
|  | C) | 256 entries | D) | 512 entries |
|  | Where is the 8086 interrupt vector table located in memory? |  |  |  |
|  | A) | In the high memory area (above 1 MB) | B) | In a dedicated register within the CPU |
|  | C) | In the data segment pointed by DS | D) | At a fixed location starting at 0000h |