UNIT 1

|  | With a Block diagram explain the architecture of 8086 microprocessor. (10 Marks) |
| --- | --- |
|  | Indicate the addressing modes of the destination operand and calculate its physical address for the following instructions of Intel 8086 microprocessor: i) ADD [DI], AL ii) SUB BLOCK, AX iii) AND [BX+DI], CL iv) ADC [BP+100H], AX Assume CS = 1000H, DS = 2000H, ES = 3000H, SS = 4000H, BX = 3FFFH, DI = 5A00H, SP = 64A0H, IP = A000H, BP = 69B0H, BLOCK = 4000H. (10 Marks) |
|  | Write an assembly level program to search the element in a given array of bytes by applying the linear search algorithm. (8 Marks) |
|  | Illustrate the following Assembler directives with proper examples: i) ASSUME ii) OFFSET iii) EQU iv) MACRO (4 Marks) |
|  | Explain the following instructions with appropriate examples: i) MOV ii) PUSH iii) IN iv) XCHG (8 Marks) |
|  | Illustrate the flag register of 8086 microprocessor with a neat diagram. (6 Marks) |
|  | Write an Assembly level program to obtain the average of two numbers. (6 Marks) |
|  | Describe the logical shift and rotate instructions with syntax. |
|  | Explain the concept of segmented memory and different types of segment registers available in 8086 Microprocessors. Show with an example how physical address is generated from logical address. (8 Marks) |
|  | What is an addressing mode? List out any 6 addressing modes and explain with examples. (8 Marks) |
|  | What is a relocation? Give the advantage of relocation. (4 Marks) |
|  | With examples, explain shift and rotate instructions available in 8086. (8 Marks) |
|  | Explain with example working of the instructions: i) XLAT ii) DAA (8 Marks) |
|  | Write an ALP to read a character from the keyboard and display it with proper messages. (4 Marks) |
|  | Explain the different general-purpose and flag registers of the 8086 microprocessor with a neat diagram. (8 Marks) |
|  | Explain the different memory addressing modes of the 8086 microprocessor with an example. (6 Marks) |
|  | Explain the following 8086 instructions with an example for each: i) DAA ii) AAA (6 Marks) |
|  | At a certain instant during the execution of a program, the 8086 processor has the following data in the registers: AX = 1234H, BX = 5678H, SI = ABCDH, DI = CDEFH, CS = 3456H, IP = 789AH, DS = ES = 4567H Identify the addressing modes used and work out the addresses of source and destination of the data when each of the following instructions is executed: i) MOV [BX], AX ii) MOV [BX+DI+3456H], 9ABCH iii) MOV AX, [9ABC] iv) MOV CX, [SI+BX] (8 Marks) |
|  | Explain the advantages of segmentation in 8086. Illustrate with an example how the physical address is generated for the code segment. (6 Marks) |
|  | Explain the working of the Bus Interface Unit (BIU) of the 8086 architecture with a diagram. (6 Marks) |
|  | Explain the following assembler directives: i) DD ii) EVEN iii) INCLUDE iv) OFFSET (4 Marks) |
|  | Develop an ALP (Assembly Language Program) to search a key element in a list of ‘n’ 8-bit numbers using the Linear Search algorithm. Display the position of the element if found; otherwise, display that the element is not found. (8 Marks) |
|  | How do various Shift Instructions work? Differentiate between arithmetic and logical shift instructions. Explain with examples. (8 Marks) |
|  | Develop an ALP with procedures to perform the following: i) Read a string from the keyboard, terminated by carriage return ii) Display the string iii) Display the length of the string (8 Marks) |
|  | Explain the flag register along with the register format with respect to Intel 8086. (8 Marks) |
|  | Develop an ALP (Assembly Language Program) to reverse a given string and check whether it is a palindrome or not. (8 Marks) |
|  | List and explain the various addressing modes available in Intel 8086. (8 Marks) |
|  | Explain the following 8086 Assembler Directives with an example for each: i) ASSUME ii) SEGMENT iii) INCLUDE iv) DB (8 Marks) |
|  | Explain the role of the following 4 directives: ASSUME, END, PROC, ENDS (4 Marks) |