MICROPROCESSOR AND EMBEDDED SYSTEMS

MULTIPLE CHOICE QUESTIONS

UNIT-2

|  | What is the purpose of the RET instruction in modular programming? |  |  |  |
| --- | --- | --- | --- | --- |
|  | A) | Loads a value into a register | B) | Returns control to the calling procedure |
|  | C) | Halts the execution of the program | D) | Calls another procedure |
|  | Which of the following is NOT an advantage of modular programming? |  |  |  |
|  | A) | Code reusability | B) | Easier debugging |
|  | C) | Increased execution time | D) | Better program organization |
|  | Which of the following statements about the CALL instruction is true? |  |  |  |
|  | A) | CALL pushes the return address onto the stack | B) | CALL cannot be used to call subroutines |
|  | C) | CALL directly modifies the IP register | D) | CALL clears the stack |
|  | Which instruction saves the current value of the flags register before calling a procedure? |  |  |  |
|  | A) | PUSH AX | B) | PUSHF |
|  | C) | POPF | D) | MOV FLAGS, AX |
|  | Which instruction is used to extend a signed byte into a word? |  |  |  |
|  | A) | CBW (Convert Byte to Word) | B) | CWD (Convert Word to Doubleword) |
|  | C) | DAA (Decimal Adjust after Addition) | D) | AAA (ASCII Adjust after Addition) |
|  | Which BIOS interrupt is used to read a key from the keyboard buffer? |  |  |  |
|  | A) | INT 10H | B) | INT 16H, AH = 00H |
|  | C) | INT 21H, AH = 01H | D) | INT 13H |
|  | To move the cursor position on the screen, which BIOS interrupt is used? |  |  |  |
|  | A) | INT 10H, AH = 02H | B) | INT 21H, AH = 02H |
|  | C) | INT 16H, AH = 00H | D) | INT 13H |
|  | Which function of INT 16H checks if a key is available? |  |  |  |
|  | A) | AH = 00H | B) | AH = 01H |
|  | C) | AH = 02H | D) | AH = 10H |
|  | Which interrupt is used to set video mode in 8086? |  |  |  |
|  | A) | INT 10H, AH = 00H | B) | INT 16H, AH = 00H |
|  | C) | INT 21H, AH = 01H | D) | INT 13H |
|  | Which instruction is commonly used to convert a hexadecimal digit to its ASCII representation in the 8086 microprocessors? |  |  |  |
|  | A) | MOV | B) | ADD |
|  | C) | OR | D) | AND |
|  | What is the ASCII value of the hexadecimal digit 0x9? |  |  |  |
|  | A) | 0x39 | B) | 0x49 |
|  | C) | 0x09 | D) | 0x1001 |
|  | If the ASCII value 0x41 (character 'A') is stored in the AL register, what will be the hexadecimal value after conversion? |  |  |  |
|  | A) | 0x1 | B) | 0xA |
|  | C) | 0x41 | D) | 0x4A |
|  | What is the hexadecimal value of the ASCII character 0x35 (character '5')? |  |  |  |
|  | A) | 0x5 | B) | 0x35 |
|  | C) | 0x0101 | D) | 0x15 |
|  | Which DOS intrupt is used to create sub directory |  |  |  |
|  | A) | AH=39h, INT21H | B) | AH=39h, INT16H |
|  | C) | AH=30h, INT 21H | D) | INT 21H |
|  | Set time interrupt from DOS interrupt will store th hour information inn which register |  |  |  |
|  | A) | CH | B) | CL |
|  | C) | DH | D) | DL |
|  | What is the use of AAM instruction in Hexa to ASCII conversion |  |  |  |
|  | A) | ASCII multiplication | B) | Convert packed BCD to unpacked BCD number |
|  | C) | Convert unpacked to packed | D) | Ascii adjust after multiplication |
|  | BIOS interrupt to set the cursor on the monitor |  |  |  |
|  | A) | AH=02H, INT 10H | B) | AH=02H, INT 16H |
|  | C) | AH=02H, INT 21H | D) | AH=01H, INT 10H |
|  | In a program, a Macro is being called 'n' times. Then how many times is the machine code generated for the same? |  |  |  |
|  | A) | 1 time | B) | 'n' times |
|  | C) | 'n-1' times | D) | ‘n+1’ times |
|  | The process of assigning a label or macroname to the string is called |  |  |  |
|  | A) | initializing macro | B) | initializing string macro |
|  | C) | defining a string macro | D) | defining a macro |
|  | Which directive is used to define a procedure in 8086 assembly language? |  |  |  |
|  | A) | FUNC | B) | PROC |
|  | C) | DEF | D) | CALL |

|  | What is the primary design philosophy of ARM embedded systems? |  |  |  |
| --- | --- | --- | --- | --- |
|  | A) | Complex Instruction Set Computing (CISC) | B) | Reduced Instruction Set Computing (RISC) |
|  | C) | Very Long Instruction Word (VLIW) | D) | Superscalar Processing |
|  | Which of the following is a feature of the ARM processor? |  |  |  |
|  | A) | Symmetric multiprocessing | B) | Harvard architecture |
|  | C) | High power consumption | D) | Scalar data types only |
|  | Which register in ARM holds the flags indicating the status of the processor? |  |  |  |
|  | A) | General-purpose register | B) | Program Counter (PC) |
|  | C) | Stack Pointer (SP) | D) | Program Status Register (PSR) |
|  | What is the function of the pinMode() function in Embedded C programming? |  |  |  |
|  | A) | Initializes serial communication | B) | Sets a digital pin as input or output |
|  | C) | Creates a delay in execution | D) | Reads analog input |
|  | Which IDE is commonly used for programming Arduino microcontrollers? |  |  |  |
|  | A) | Keil uVision | B) | Arduino IDE |
|  | C) | MPLAB X | D) | Eclipse |
|  | What is the size of the general-purpose registers in ARM processors? |  |  |  |
|  | A) | 8 bits | B) | 16 bits |
|  | C) | 32 bits | D) | 64 bits |
|  | Which function is used to generate a delay in Embedded C? |  |  |  |
|  | A) | pinMode() | B) | delay() |
|  | C) | digitalWrite() | D) | Serial.begin() |
|  | How does the 7-segment display represent numbers? |  |  |  |
|  | A) | Using rows and columns of LEDs | B) | By turning on specific segments of LEDs |
|  | C) | Through alphanumeric LCD cells | D) | By activating relays |
|  | Which of the following peripherals can be interfaced with a microcontroller? |  |  |  |
|  | A) | LED | B) | Registers |
|  | C) | Segment | D) | diode |
|  | What is the main purpose of the Arduino Due microcontroller board? |  |  |  |
|  | A) | High-performance desktop computing | B) | General-purpose embedded applications |
|  | C) | High-speed graphics rendering | D) | Quantum computing |
|  | Which of the following is the primary advantage of using ARM-based microcontrollers in embedded systems? |  |  |  |
|  | A) | High power consumption | B) | Support for only digital peripherals |
|  | C) | Low power consumption with high performance | D) | Limited peripheral interfacing options |
|  | Which built-in Arduino function is used for serial communication? |  |  |  |
|  | A) | digitalWrite() | B) | Serial.begin() |
|  | C) | analogRead() | D) | pinMode() |
|  | What is the typical clock speed of an ARM Cortex-M microcontroller? |  |  |  |
|  | A) | 1MHz | B) | 50 to 100 MHz |
|  | C) | 1.5GHz | D) | 1 GHz |
|  | In Embedded C, what does the function digitalWrite() do? |  |  |  |
|  | A) | Sets a digital pin as input | B) | Reads the state of a digital pin |
|  | C) | Writes a HIGH or LOW value to a digital pin | D) | Configures PWM on a pin |
|  | What is the resolution of the ADC on the Arduino Due? |  |  |  |
|  | A) | 8-bit | B) | 10-bit |
|  | C) | 12-bit | D) | 16-bit |
|  | Which of the following displays supports scrolling and flickering? |  |  |  |
|  | A) | Seven-segment display | B) | 16x2 LCD |
|  | C) | OLED | D) | VGA monitor |
|  | How does a stepper motor differ from a DC motor? |  |  |  |
|  | A) | It operates on AC power | B) | It moves in discrete steps |
|  | C) | It has no rotor | D) | It cannot reverse direction |
|  | Which Arduino board pin is used for analog input? |  |  |  |
|  | A) | D0-D7 | B) | A0-A5 |
|  | C) | PWM | D) | Vin |
|  | What is the typical use of a 4x4 keypad in embedded systems? |  |  |  |
|  | A) | Memorystorage | B) | Input device for user interaction |
|  | C) | Powerregulation | D) | Debugging tool |
|  | Which ARM instruction type is most commonly used for data transfer? |  |  |  |
|  | A) | Branch instructions | B) | Load/Store instructions |
|  | C) | Arithmeticinstructions | D) | Logical instructions |